|  | WSC - HOT WORK PERMIT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Warning! |  |  | HOT WORK IN PROGRESS WATCH FOR FIRE! |  |  |
|  | Instructions |  |  |  | Required Precautions Checklist (must be retained as a record of hot work activity for minimum of 6 months) |  |
|  | - Person doing hot work: indicate time started and post permit at hot work location. After hot work, indicate time completed and leave permit posted for fire watch. - Fire Watch: Prior to leaving area, do final inspection, sign, leave permit posted and Inform permit authoriser. - Monitor: After 6 hours, do final inspection, sign and return to Club Administrator. |  |  |  |  |  |
|  |  |  |  |  | Fire extinguisher available at location Within 10m radius of work: Floors, work level and below, cleaned or protected All other combustibles removed or shielded from sparks: - Clean horizontal surfaces (eg building structures, equipment, ducts, cable trays etc) above and below where possible - Remove flammable liquids, dust, lint, combustible waste, oil deposits etc. where possible. - If removal/cleaning is impractical, protect with fire-retardant covers, or shield with fire-retardant guards and/or curtains. Transmission or conveying of sparks to adjacent areas eliminated or protected - Tightly cover wall/floor openings with fire retardant material - Where opening cannot be sealed, suspend fire-retardant tarpaulins to help protect areas beneath - Isolate or shutdown fans to prevent the capturing and conveying of sparks to other areas. Explosive atmosphere eliminated or not present Work on walls, ceilings or enclosed equipment: Construction materials verified as non-combustible and without combustible covering or insulation Combustibles on other side of walls relocated or protected Enclosed equipment cleaned and protected from all combustibles. Containers purged of flammable liquids/vapours Fire watch/hot work area monitoring requirements: Continuous fire watch provided during and for at least 30 minutes after hot work, including all breaks. Fire watch supplied with suitable fire extinguisher Fire watch trained in use of extinguisher Area to be monitored hourly for a minimum of 6 hours after job is completed, or longer if required by officer authorising permit. Other precautions that may be required: Area smoke or heat detector disabled to eliminate false trip Other: ____________________________________________ Comment by authorising officer: |  |
|  | Who, When and Where? |  |  |  |  |  |
|  | Hot work being done by |  |  |  |  |  |
|  | Employee Contractor |  |  |  |  |  |
|  | Date |  |  |  |  |  |
|  | Location |  |  |  |  |  |
|  | Nature of job |  |  |  |  |  |
|  | Name of person(s) doing Hot Work |  |  |  |  |  |
|  | Signature of Permit-Authorising Officer |  |  |  |  |  |
|  | Time Started | Time finished |  |  |  |  |
|  | Work area and all adjacent areas to which sparks and heat might have spread were inspected during the fire watch period and were found fire safe. |  |  |  |  |  |
|  | Signature Fire Watch/Monitor |  | Time |  |  |  |